PRACTICAL 2

Aim: To Explore and Manage Raster data

- Adding Raster Layer

Procedure:

- From menu bar select Layer → Add Layer → Add Raster Layer

- Select Gridded Population of the World (GPW) v3 dataset from Columbia University, Population Density Grid for the entire globe in ASCII format and for the year 1990 and 2000. “\GIS_Workshop\Practicals\Practical_02\A\Data\gl_gpwv3_pdens_90_ascii_one\glds90ag60.asc” “\GIS_Workshop\Practicals\Practical_02\A\Data\gl_gpwv3_pdens_90_ascii_one\glds00ag60.asc”

- Go to Project → Properties OR Press the Set CRS option on bottom right corner.

- Select WGS 84 EPSG: 4326 and Press OK

- Raster Styling and Analysis

Procedure:

- To start with analysis of population data, convert the pixel from grayscale to Color. Select “glds90ag60.asc” Layer form layer Pane → select property OR double click on it. Select Symbology

- Select Render Type as “Singleband pseudocolor”. Min=0 and Max=240. Select the Color Ramp. Press “APPLY” .

- Repeat the same for “glds00ag60.asc” Layer

- Layer output after applying style.

- Go to Raster → Raster Calculator

- Put the expression "glds00ag60@1" - "glds90ag60@1"

- Select the output file location & name as Pop_Diff and Press OK.

- Remove the other two layers i.e. glds00ag60.asc and glds90ag60.asc

- Double click on pop_diff layer.  Select Symbology

- Set Render Type to “Single band Pseudo color”, Interpolation as Discrete, and remove all classification and add as shown in figure above using button. After all settings press “OK”.

- Layer will appear like:

- Raster mosaicking and clipping

Procedure:

- Go to Layer → Add Layer → Add Raster Layer.

- Select the following “.tif” raster images

FAS_India1.2018349.terra.367.2km.tif FAS_India2.2018349.terra.367.2km.tif FAS_India3.2018349.terra.367.2km.tif FAS_India4.2018349.terra.367.2km.tif

- Raster window click Add.

- Go to Raster → Miscellaneous → Merge

- In the Merge dialog window. Select three dots Input Layers. Select all layers and Press OK. In Merge dialog window select a file name and location to save merged images.

- Save the file to a location with the name as Merge_Files.tif ➢ Press Run and after completion of operation close the Merge window dialog box.

- You can now deselect individual layers from layer pane and only keep the merged raster file.

- Go to Layer→ Add Vector Layer → Select \GIS_Workshop\Practicals\Practical_02\C\IndiaAdminBoundry\IND_adm0.shp file.  From layer properties → select → select any one of the following

- The result will be

- Go to Raster → Extraction → Clip Raster by Mask Layer

- Select the merge raster image as input and Ind_adm0 as mask layer.  Select a file name and location for clipped raster as /Practical_02/C/Clipped_File.tif.

- Press RUN

After Clipping

Output: Raster data has been explored and managed successfully.